Product Name: 9/16” EZ Stab Tier Drop System

Manufacturer: CertainTeed Architectural

# SECTION 09 53 00 (095300) – ACOUSTICAL CEILING SUSPENSION ASSEMBLIES

## PART 1 – GENERAL

  - RELATED DOCUMENTS

A. 	Drawings and general provisions of the contract apply to this section. This includes General and Supplementary Conditions of Division 01 (1) Specification Sections.

  - SUMMARY

- Section includes acoustical ceiling panel suspension systems for ceilings

- Related Sections

- Section 09 20 00 (09250) – Plaster and Gypsum Board

- Section 09 50 00 (09500) – Ceilings

- Section 09 51 00 (09510) – Acoustic Ceilings

- Division 23 (15) – Heating, Ventilating and Air Conditioning (HVAC)

- Division 26 (16) – Electrical

  - REFERENCES

- ASTM A641 – Specification for Steel Sheet, Zinc-Coated (galvanized) Carbon Steel Wire

- ASTM A653 – Standard Specification for Steel Sheet, Zinc-Coated (Galvanized) or Zinc-Iron Alloy-Coated (Galv-annealed) by the Hot-Dip Process

- ASTM A1008 – Standard Specification for Steel, Sheet, Cold Rolled, Carbon, Structural, High-Strength Low-Alloy and High-Strength Low-Alloy with Improved Formability

- ASTM C635 – Standard Specification for Metal Suspension Systems for Acoustic Tile and Lay-in Panel Ceilings

- ASTM C636 – Recommended Practice for Installation of Metal Ceiling Suspension Systems for Acoustical Tile and Lay-in Panel Ceilings

- ASTM D610 – Standard Test Method for Evaluating Degree of Rusting on Painted Steel Surfaces

- ASTM E580 – Practice for Application of Ceiling Suspension Systems for Acoustic Tile and Lay-in Panels in Areas Requiring Seismic Restraint

- CISCA (Ceilings & Interior Systems Construction Association) – Ceilings Systems Handbook

- CISCA (Ceilings & Interior Systems Construction Association) – Acoustical Ceilings – Use and Practice

- CISCA (Ceilings & Interior Systems Construction Association) – Guidelines For Seismic Restraint Direct Hung Suspended Ceiling Assemblies

- ISO 14025 - Environmental Labels and Declarations -- Type III Environmental Declarations -- Principles and Procedures

- International Code Council-Evaluation Services Report - Seismic Engineer Report

- Standard Method for the Testing and Evaluation of Volatile Organic Chemical Emissions from Indoor Sources Using Environmental Chambers Version 1.2

  - California Dept. of Public Health CDPH/EHLB/Standard Method v1.2, 2017

- Health Product Declaration Standard v2.0 – hpdcollaborative.org

- International Living Future Institute – LBC Declare Label

  - SUBMITTALS

- Product Data

- Submit manufacturer’s published technical information for each product indicated

- Shop Drawings

- Submit reflected ceiling plans drawn to scale prescribed by Architect

  - Include coordinated penetrations and ceiling-mounted items

  - Include any necessary details or drawings from the manufacturer regarding recommended installation

- Samples

- Submit 12 inch long samples of suspension system components, including main runner, cross tee, wall angle

- Submit representative manufacturer’s sample of each suspension member indicated

- Certifications

# Provide manufacturer’s written certification that products submitted meet or exceed all specified requirements

- Provide third party verified life cycle information with published environmental product declaration (EPD)

  - Per ISO 14025 Environmental Labels and Declarations - Type III Environmental Declarations - Principles and Procedures

  - QUALITY ASSURANCE

- Source Limitations

- Acoustical Suspension System

  - Obtain all acoustical ceiling framing components through one source from a single manufacturer

- Installer Qualifications

- Must be experienced in the installation of systems similar to those specified herein

  - DELIVERY, STORAGE AND HANDLING

- Delivery of acoustical suspension system will be in the original unopened packages with the manufacturer’s label intact

- Handling and storage should be in accordance with the manufacturer’s Safety Data Sheets (SDS)

- Product cartons should be handled carefully to avoid damage

  - COORDINATION

- Coordinate the installation of the acoustical suspension system with any and all trades whose work is impacted by that installation

  - EXTRA MATERIALS

- Provide extra materials in the manufacturer’s unopened packaging, with the manufacturer’s label intact, as detailed below

- Suspension System Components – Minimum 5% of each type installed

- PART 2 - PRODUCTS

2.1 MANUFACTURER

- CertainTeed Architectural

- Address: 20 Moores Road, Malvern, PA 19355

- Telephone: 800-233-8990

- Web: www.certainteed.com/architectural

2.2 SUSPENSION SYSTEM

- Manufacturer: CertainTeed Architectural

- Product

  - Name: [9/16” EZ Stab Tier Drop System]

- Physical Characteristics

- Structural Classification: [Intermediate Duty, Heavy Duty] (per ASTM C635)

- Double web design manufactured of hot-dipped galvanized steel [G30]

- Cross tees feature EZ Clip – reduced insertion force, easy removability

- Main runners and cross tees feature double-web stitching to increase torsional rigidity, reduced grid rotation or bending

- Centering tabs on all tees to prevent panel shifting and visual gaps when installing square edge panels in narrow grid

- Unmitered intersections for flexible cross tee placement

- 9/16” face with 3/16” wide center bulb

- Tee height: [1-3/4”]

- Flange Size: [9/16”]

- Standard Color: [White]

- Recycled Content:

  - Main Runners: 86%

    - 9% (pre-consumer)

    - 77% (post-consumer)

  - Cross Tees: 86%

    - 9% (pre-consumer)

    - 77% (post-consumer)

  - Wall Angle: 88%

    - 19% (pre-consumer)

    - 69% (post-consumer)

  - Shadow Molding: 53% - 88%

    - SM1000, SM1020: 88%

      - 19% (pre-consumer)

      - 69% (post-consumer)

    - SM1030, SM1040, SM1050, SM1060: 53%

      - 9% (pre-consumer)

      - 44% (post-consumer)

- Components

- Main Runners

  - Size: [144”]

- Cross Tees

  - Size: [4”, 6”, 24”, 30”, 48”, 60”, 72”, 96”]

- Edge Molding

  - Type: [angle, shadow molding]

  - Profile: As selected by the Architect

  - Size: [120”, 144”]

- Attachment Devices: Anchors sufficient for five-times design load indicated in ASTM C635 (Table 1). Wire for hangers of size and type to suit intended application, complying with ASTM A641, Class 1 zinc coating, not less than 12 gauge

  - Seismic Restraints: Pursuant to ASTM E580 and local code requirements

  - ICC-ES Evaluation Service Report (ESR-3336)

    - Suspended Ceilings Framing Systems and Seismic Perimeter Clip

    - City of Los Angeles Building Code compliant via ESR-3336 LABC Supplement

- Independent Certifications

  - VOC content

    - Third-party certification of compliance

      - Per California Dept. of Public Health CDPH/EHLB/Standard Method v1.2, 2017

  - Environmental Product Declaration

    - Third-party verified Type III Environmental Product Declaration

      - Per ISO 14025 - Environmental Labels and Declarations - Type III Environmental Declarations -- Principles and Procedures

  - Health Product Declaration

    - Per Health Product Declaration Standard v2.0

      - hpd-collaborative.org

  - Living Building Challenge Declare Label

    - LBC Red List Free, per International Living Future Institute

## PART 3 – EXECUTION

3.1 EXAMINATION

- Ascertain acceptability of substrates and building conditions under which the ceiling system is to be installed. Do not proceed with the installation until any and all unacceptable conditions have been rectified.

3.2 INSTALLATION

- Install the ceiling system in accordance with the following:

- Manufacturer’s printed instructions

  - Available online at www.certainteed.com/architectural

- ASTM C636

- Ceilings & Interior Systems Construction Association (CISCA) recommendations

- Applicable local code requirements

- Approved shop drawings

# END OF SECTION

CertainTeed Architectural shall be held harmless for any damages resulting from the use of this specification guide